3rd Year Electronic Engineering Project Proposals (Adam Dunworth)

2025/26

Data Centre Liquid Cooling Leak Detection System

The student will develop platform for liquid cooling leak detection in data centres. Using an MSP430 microcontroller, multiple sensors (pressure, flow, conductivity, moisture etc.) are integrated into a combined algorithm capable of identifying and classifying leak events such as drips, seepages, or bursts. The student will implement communication via MQTT for remote monitoring on a dashboard, while a local LCD interface provides configuration and live diagnostics. The system is designed with fault-tolerant features to handle sensor failures or noise. This project demonstrates how multi-sensor embedded systems can enhance operational reliability in mission-critical cooling applications.

Microcontroller-Based Digital Overcurrent Protection Relay

The student will develop a prototype digital overcurrent protection relay using an MSP430 microcontroller. The relay will measure fault current through a conditioned CT signal and apply IEC standard inverse curves, including SI, VI, EI, and LTI. Users can configure pickup current and time multipliers using a keypad and view status, fault logs, and diagnostics on an LCD interface. Events and real-time measurements are communicated to external monitoring systems via MQTT. The system is designed with fault-tolerant features to handle CT sensor or harmonics. The project demonstrates how embedded systems can implement modern, cost-effective protection solutions.

Grid Frequency-Based Demand Response Controller

The student will develop a grid frequency-based demand response controller using an MSP430 microcontroller. The system measures frequency using a zero-crossing detection circuit and applies shedding or restoration logic when deviations indicate supply-demand imbalance. Loads are managed by priority, with critical and non-critical categories configurable via the user interface. An LCD displays frequency, load states, and thresholds, while MQTT integration with monitoring systems. Hysteresis control ensures stable operation without rapid switching. The project demonstrates how embedded controllers can support smart grid integration by automatically balancing load in response to real-time grid conditions.

MSP430-Based Maritime Wind and Weather Station for Nearshore Sea Buoys with Remote Data Logging

The student will develop a maritime weather station for sea buoys using an MSP430 microcontroller, designed to monitor wind speed, wind direction, rainfall, temperature, humidity, and pressure. Sensor readings are processed in real-time to filter noise from waves and rough sea conditions. The system transmits data wirelessly via LoRa to a shore-based server where readings are logged in a SQL database for historical analysis and trend visualization. A local LCD provides live diagnostics and battery status. The project demonstrates how embedded maritime sensing, low-power design, can support navigation safety, weather monitoring, and marine research.